EXP:1

% Simplified illustrative model; not a full cellular-network simulation.

clear; clc; close all;

% Illustrative system parameters

totalBandwidthHz = 1e6;       % Total bandwidth: 1 MHz fdmaChannelHz    = 200e3;     % FDMA channel width: 200 kHz tdmaSlots        = 8;         % Number of TDMA slots per frame processingGain   = 64;        % CDMA processing gain targetSIRdB      = 7;         % Minimum acceptable CDMA SIR

% Capacity estimates under the stated assumptions fdmaUsers = floor(totalBandwidthHz / fdmaChannelHz); tdmaUsers = tdmaSlots;

targetSIR = 10^(targetSIRdB / 10);

cdmaUsers = floor(1 + processingGain / targetSIR);

fprintf('Illustrative maximum supported users\n'); fprintf('FDMA: %d users\n', fdmaUsers); fprintf('TDMA: %d users\n', tdmaUsers); fprintf('CDMA: %d users at a target SIR of %.1f dB\n\n', ...

cdmaUsers, targetSIRdB);

% CDMA SIR as the number of simultaneous users increases K = 2:30;

sirLinear = processingGain ./ (K - 1); sirDb = 10 * log10(sirLinear);

% Display a comparison table method = ["FDMA"; "TDMA"; "CDMA"];

supportedUsers = [fdmaUsers; tdmaUsers; cdmaUsers];

results = table(method, supportedUsers); disp(results);

% Plot estimated user capacity figure;

bar(categorical(method), supportedUsers);

ylabel('Illustrative maximum supported simultaneous users'); title('FDMA, TDMA and CDMA: Capacity Estimate'); grid on;

% Plot estimated CDMA SIR as users are added figure;

plot(K, sirDb, 'o-', 'LineWidth', 1.5); hold on;

yline(targetSIRdB, '--r', 'Target SIR'); xlabel('Number of simultaneous CDMA users (K)'); ylabel('Estimated SIR (dB)');

title('Estimated CDMA SIR versus Number of Users'); grid on; hold off;

EXP:2

% Experiment 2: simplified hexagonal coverage and handoff clear; clc; close all;

% Baseline assumptions serviceAreaKm2 = 5; radiiKm = [1.0 0.5]; usersPerCell = 30; hexArea = @(R) (3*sqrt(3)/2) .* R.^2;

cellAreaKm2 = hexArea(radiiKm); cellCount = ceil(serviceAreaKm2 ./ cellAreaKm2); estimatedUsers = cellCount .* usersPerCell; capacityTable = table(radiiKm(:), cellAreaKm2(:), cellCount(:), ...     repmat(usersPerCell,numel(radiiKm),1), estimatedUsers(:), ...

'VariableNames', {'Radius_km','CellArea_km2','EstimatedCells', ...

'UsersPerCell','IllustrativeUsers'});

disp(capacityTable);

% Plot a 7-cell cluster: central cell plus six neighbors

R = radiiKm(1); centers = [0 0; 1.5*R sqrt(3)*R/2; 1.5*R -sqrt(3)*R/2; ...

0 sqrt(3)*R; 0 -sqrt(3)*R; -1.5*R sqrt(3)*R/2; ...

-1.5*R -sqrt(3)*R/2];

theta = (0:6)*pi/3; % flat-top hexagon vertices figure; hold on; axis equal; grid on; for k = 1:size(centers,1)     plot(centers(k,1)+R*cos(theta), centers(k,2)+R*sin(theta), 'k-');     text(centers(k,1),centers(k,2),sprintf('Cell %d',k), ...

'HorizontalAlignment','center'); end

xlabel('x (km)'); ylabel('y (km)');

title('Idealized Seven-Cell Hexagonal Cluster');

hold off;

% Mobile moves across the boundary between centers at x=0 and x=1.5R mobileX = linspace(-0.2*R,1.7*R,300); mobileY = zeros(size(mobileX)); d1 = hypot(mobileX, mobileY); d2 = hypot(mobileX-1.5*R, mobileY); servingCell = ones(size(mobileX)); servingCell(d2 < d1) = 2; handoffIndex = find(diff(servingCell) ~= 0,1,'first') + 1; figure; hold on; grid on; plot(mobileX,d1,'LineWidth',1.5); plot(mobileX,d2,'LineWidth',1.5); if ~isempty(handoffIndex)     xline(mobileX(handoffIndex),'--r','Handoff decision'); end xlabel('Mobile x-position (km)'); ylabel('Distance to cell center (km)');

legend('Cell 1','Cell 2','Location','best'); title('Nearest-Center Handoff Approximation'); hold off; figure; stairs(mobileX,servingCell,'LineWidth',1.5); grid on; xlabel('Mobile x-position (km)'); ylabel('Serving-cell ID'); title('Serving Cell Along the Mobile Path');

if ~isempty(handoffIndex)     fprintf('Geometric handoff at x = %.3f km\n',mobileX(handoffIndex)); end

EXP:3

clear; clc; close all;

% Representative bands and link-budget assumptions freqGHz = [0.7 3.5 28]; freqMHz = freqGHz * 1000; bandName = ["Sub-1 GHz"; "Mid-band"; "mmWave"]; txPowerdBm = 30; gtDbi = 0; grDbi = 0; distanceKm = logspace(log10(0.1),log10(2),200); distanceAtPointKm = 0.5;

% Free-space path loss (d in km, f in MHz)

fspl = @(fMHz,dKm) 32.44 + 20*log10(fMHz) + 20*log10(dKm);

lossAtPoint = fspl(freqMHz,distanceAtPointKm); rxAtPoint = txPowerdBm + gtDbi + grDbi - lossAtPoint; linkTable = table(bandName,freqGHz(:),lossAtPoint(:),rxAtPoint(:), ...     'VariableNames',{'Band','Frequency_GHz','FSPL_dB','EstimatedRx_dBm'}); disp(linkTable);

% Path loss and received power over distance

figure; tiledlayout(1,2); nexttile; hold on; grid on; for k=1:numel(freqMHz)     plot(distanceKm,fspl(freqMHz(k),distanceKm),'LineWidth',1.5); end

set(gca,'XScale','log'); xlabel('Distance (km)'); ylabel('Path loss (dB)'); legend(bandName,'Location','northwest'); title('Free-Space Path Loss'); nexttile; hold on; grid on; for k=1:numel(freqMHz)     rx = txPowerdBm + gtDbi + grDbi - fspl(freqMHz(k),distanceKm);     plot(distanceKm,rx,'LineWidth',1.5); end

set(gca,'XScale','log'); xlabel('Distance (km)'); ylabel('Estimated received power (dBm)'); legend(bandName,'Location','southwest'); title('Illustrative Received Power');

% Simple first-tier co-channel reuse estimate

cellRadiusKm = 0.5; reuseN = [1 3 7 12]; pathLossExponent = 3.5;

reuseDistance = sqrt(3*reuseN) * cellRadiusKm; sirLinear = (reuseDistance/cellRadiusKm).^pathLossExponent / 6; sirDb = 10*log10(sirLinear); reuseTable = table(reuseN(:),reuseDistance(:),sirDb(:), ...

'VariableNames',{'ReuseClusterN','ApproxInterfererDistance_km','ApproxSIR_dB'}); disp(reuseTable);

figure; plot(reuseN,sirDb,'o-','LineWidth',1.5); grid on; xlabel('Reuse cluster size N'); ylabel('Approximate SIR (dB)'); title('Simplified Co-Channel SIR versus Reuse Cluster Size');

fprintf('At %.2f km, estimated received powers are:\n',distanceAtPointKm); disp(table(bandName,rxAtPoint(:),'VariableNames',{'Band','Rx_dBm'}));

EXP:5

clc; clear; close all;

BW1 = 20; % Component carrier bandwidth (MHz)

SE = 4; % Spectral efficiency (bits/s/Hz)

Ncc = 1; % Number of component carriers

BWagg = BW1*Ncc;

Throughput = BWagg*SE; % Mbps for MHz × bits/s/Hz

disp('Single Component Carrier')

disp(['Bandwidth = ',num2str(BWagg),' MHz'])

disp(['Estimated Throughput = ',num2str(Throughput),' Mbps'])

figure

bar(Ncc,Throughput)

grid on

xlabel('Number of Component Carriers')

ylabel('Estimated Throughput (Mbps)')

title('LTE-Advanced Single Carrier Throughput')

clc; clear; close all;

BWcc = 20; % MHz per component carrier

SE = 4; % bits/s/Hz

CC = [1 2 3]; % Number of component carriers

AggBW = BWcc .* CC; % MHz

Throughput = AggBW .* SE; % Mbps

disp('CC Aggregated BW (MHz) Estimated Throughput (Mbps)')

disp([CC' AggBW' Throughput'])

figure

bar(CC,Throughput)

grid on

xlabel('Number of Aggregated Component Carriers')

ylabel('Estimated Throughput (Mbps)')

title('LTE-Advanced Carrier Aggregation Throughput')

clc; clear; close all;

BW = [5 10 15 20]; % Carrier bandwidths (MHz)

SE = 4; % bits/s/Hz

TP = BW .* SE; % Estimated throughput (Mbps)

disp('Bandwidth (MHz) Throughput (Mbps)')

disp([BW' TP'])

figure

plot(BW,TP,'o-','LineWidth',2)

grid on

xlabel('Carrier Bandwidth (MHz)')

ylabel('Estimated Throughput (Mbps)')

title('Throughput vs Component Carrier Bandwidth')

EXP:6

clc; clear; close all;

SNR_dB = 0:2:20;

BER_SISO = zeros(size(SNR_dB));

BER_MIMO = zeros(size(SNR_dB));

for k = 1:length(SNR_dB)

N = 10000;

bits = randi([0 1],N,1);

tx = 2*bits-1;

snrLin = 10^(SNR_dB(k)/10);

noise = randn(N,1)/sqrt(2*snrLin);

rx = tx + noise;

bits_hat = real(rx)>0;

BER_SISO(k) = mean(bits_hat~=bits);

% Simple 2x2 MIMO equivalent link

H = (randn(2,2)+1j*randn(2,2))/sqrt(2);

h_eff = sqrt(sum(abs(H(:)).^2)/2);

noise2 = randn(N,1)/sqrt(2*snrLin);

rx2 = h_eff*tx + noise2;

bits_hat2 = real(rx2)>0;

BER_MIMO(k) = mean(bits_hat2~=bits);

end

figure

semilogy(SNR_dB,BER_SISO,'o-','LineWidth',2); hold on

semilogy(SNR_dB,BER_MIMO,'s-','LineWidth',2)

grid on

xlabel('SNR (dB)')

ylabel('Bit Error Rate (BER)')

title('SISO and MIMO BER Performance')

legend('SISO','MIMO')

clc; clear; close all;

SNR_dB = 0:2:20;

Reliability = zeros(size(SNR_dB));

for k = 1:length(SNR_dB)

N = 10000;

bits = randi([0 1],N,1);

tx = 2*bits-1;

snrLin = 10^(SNR_dB(k)/10);

noise = randn(N,1)/sqrt(2*snrLin);

rx = tx + noise;

bhat = real(rx)>0;

BER = mean(bhat~=bits);

Reliability(k) = 1-BER;

end

figure

plot(SNR_dB,Reliability,'o-','LineWidth',2)

grid on

xlabel('SNR (dB)')

ylabel('Reliability (1-BER)')

title('Communication Reliability vs SNR')

clc; clear; close all;

N = 8; % Antenna elements

theta = -90:1:90; % Observation angle

steer = [-60 -30 0 30 60]; % Steering directions

gain = zeros(size(steer));

for k = 1:length(steer)

% Simplified normalized array response at the intended direction

gain(k) = 10*log10(N) - 0.01*abs(steer(k));

end

figure

plot(steer,gain,'o-','LineWidth',2)

grid on

xlabel('Steering Angle (Degrees)')

ylabel('Beamforming Gain (dB)')

title('Beamforming Gain for Different Steering Directions')

EXP:7

clc; clear; close all;

Nsc = 24; % Simplified number of subcarriers

Nsym = 14; % OFDM symbols per slot

grid = zeros(Nsym,Nsc);

% Simplified DL assignment: symbols 1-10

grid(1:10,:) = 1;

% Simplified UL assignment: symbols 11-14

grid(11:14,:) = 2;

figure

imagesc(grid)

grid on

xlabel('Subcarrier Index')

ylabel('OFDM Symbol Index')

title('Simplified 5G NR Time-Frequency Resource Grid')

clc; clear; close all;

scs = [15 30 60 120 240]; % kHz

mu = 0:4;

slot = [1 0.5 0.25 0.125 0.0625]; % ms

figure

plot(scs,slot,'o-','LineWidth',2)

grid on

xlabel('Subcarrier Spacing (kHz)')

ylabel('Slot Duration (ms)')

title('5G NR Slot Duration for Different Subcarrier Spacing')

figure

bar(scs)

grid on

set(gca,'XTickLabel',mu)

xlabel('Numerology Index (\mu)')

ylabel('Subcarrier Spacing (kHz)')

title('5G NR Subcarrier Spacing for Different Numerologies')

clc; clear; close all;

symbols = 1:14;

DL = [1 1 1 1 1 1 1 1 1 1 0 0 0 0];

UL = [0 0 0 0 0 0 0 0 0 0 1 1 1 1];

figure

plot(symbols,DL,'o-','LineWidth',2); hold on

plot(symbols,UL,'s-','LineWidth',2)

grid on

xlabel('OFDM Symbol Index')

ylabel('Allocation Status')

title('Simplified Downlink and Uplink Assignment')

legend('Downlink','Uplink')